| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90! - Use commands G0 and G1! - Enter program %1 into your computer and simulate machining! - Consult your teacher for removing of any errors! |  |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1800 1/min; 800 1/ min Feed rate(F): 80 mm/min Tools: T1 pre drill Ø 3 T2 drill Ø 10 | M03 spindle ON, clockwise rotation M08 cooling medium ON M09 cooling medium OFF M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed |