| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numericlly Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and for centre point path of the tool. - Use commands G00, G01, G02 and G03. - Enter the main program %8 and both subroutines L110 / L111 into your computer and simulate machining. - Consult your teacher for removing of any failures.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed(S): 2500 1/ min Feed rate(F): 60 mm/min Milling depth: 3 mm Tool: T1 cutter Ø6 | M03 spindle ON, clockwise rotation M30 main program STOP M17 subroutine END G01 straight line with given feed G02 circular are clockwise G03 circular are anticlockwise |