| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Please program machining of work piece with G90, with tool path correction and the circle – like depression as well as rectangle depression with corresponding cycles! - Use commands G00, G01, G02, G03, G40, G41, G42 and variables for the depressions! - Enter program %10 into your computer and simulate machining! - Consult your teacher for removing of any failures!. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation Speed(S): 1500 1/min 2500 1/ min Feed rate(F): 60 mm/min Milling depth: 6 mm Tools: T1 cutter Ø30, T2 cutter Ø12 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapit feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circular are anticlockwise R01, R02, …. Parameters of cycles |