# Bilingualer Fachunterricht an Berufsbildenden Schulen

Unterrichtsmodul

Berufsfeld:  	Metalltechnik

COMPUTER

NUMERICALLY CONTROLLED

BORING- and MILLING WORK

ACTION PROGRAMME OF THE EUROPEAN UNION

LEONARDO DA VINCI

PROJEKT NO.: 2002 LA 112 628 BILVOC

Language Competence Through

Bilingual Teaching at Vocational Colleges

###### Teaching Modules

Date of production			09/2005

Subject:					Metal technology

Topic:					Programming of CNC milling machines

Title: 						Boring - and milling work

Target Group: 				Any group of metal technology students

Prior Knowledge:				Basic knowledge of computer numerical control

Level of Language Skills:		Way stage (A2)

Follow - Ups:	Programming exercises for machining of work    piece

Drilling- and milling work

Objectives:					To understand the importance of precautions

Benefits:		Avoiding errors

Avoiding machine defects caused by faulty programmes

Teaching - module: Programming of computer controlled milling machines (CNC) and creation of a workshop drawing with the Computer (CAD)

12 lessons of 90 mins with the structure:

| Teaching Time | Contents | Activities | Media | Activities |
| --- | --- | --- | --- | --- |
| 10 minutes 35 minutes 20 minutes 10 minutes 5 minutes 5 minutes 5 minutes | ### Introduction into conceptual formulation Workshop drawing CNC program Discussion of results Input of program Simulation of machining evaluation | Teacher – students discussion Discussion about work sheet Work with dictionary Students work on pc 4 groups working with the work sheet 1 group present their results Enter the program on Pc Operation of software Teacher – students discussion Discussion of errors | worksheet dictionary worksheet Computer with CAD - software worksheet solutions sheet transparency, projector Computer with CNC - software Computer with CNC - software Computer with CNC - software solutions sheet | Lecture by teacher Group discussion Individual work on pc with teacher Creation of CNC programme on the solution sheet Students discussion of results Input of programme into Pc Operation of Pc Individual work Students presentation Correction of errors |
| 90 minutes |  |  |  |  |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90! - Use commands G0 and G1! - Enter program %1 into your computer and simulate machining! - Consult your teacher for removing of any errors! |  |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1800 1/min; 800 1/ min Feed rate(F): 80 mm/min Tools: T1 pre drill Ø 3 T2 drill Ø 10 | M03 spindle ON, clockwise rotation M08 cooling medium ON M09 cooling medium OFF M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Please program machining of work piece with G90. - Use commands G0 and G1, L83 and L92. - Enter program %9 into your computer and simulate machining. - Consult your teacher for removing of any failures. |  |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1800 1/min; 800 1/ min Feed rate(F): 80 mm/min Tools: T1 pre drill Ø 3 T2 drill Ø 10 | M03 spindle ON, clockwise rotation M08 cooling medium ON M09 cooling medium OFF M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and for centre point path of the tool! - Use commands G0 and G1! - Enter program %2 into your computer and simulate machining! - Consult your teacher for removing of any errors! |  |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1200 1/ min Feed rate (F): 60 mm/min Milling depth: 6 mm Tool: T1 cutter Ø10 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and for centre point path of the tool. - Use commands G0, G01, G02 and G03. - Enter program %3 into your computer and simulate machining. - Consult your teacher for removing of any errors. |  |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1200 1/ min Feed rate (F): 60 mm/min Milling depth: 6 mm Tool: T1 cutter Ø8 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circular are anticlockwise |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and for centre point path of the tool. - Use commands G00, G01, G02 and G03. - Enter program %4 into your computer and simulate machining. - Consult your teacher for removing of any errors.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 2500 1/ min Feed rate(F): 60 mm/min Milling depth: 6 mm Tool: T1 cutter Ø6 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circular are anticlockwise |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numericlly Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and for centre point path of the tool. - Use commands G00, G01, G02 and G03. - Enter program %5 into your computer and simulate machining. - Consult your teacher for removing of any errors.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation Speed (S): 2500 1/ min Feed rate(F): 60 mm/min Milling depth: 3 mm Tool: T1 cutter Ø6 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circular are anticlockwise |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and with tool path correction. - Use commands G00, G01, G02, G03, G40, G41 and G42. - Enter program %6 into your computer and simulate machining. - Consult your teacher for removing of any errors.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1200 1/ min Feed rate(F): 60 mm/min Milling depth: 10 mm Tools: T1 cutter Ø 30 G40 Tool path correction OFF G41 Tool path correction ON and left ##### G42 Tool path correction ON and right | M03 spindle ON, clock wise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circular are anticlockwise |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and with tool path correction. - Use commands G00, G01, G02, G03, G40, G41 and G42. - Enter program % 7 into your computer and simulate machining. - Consult your teacher for removing of any errors.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed (S): 1200 1/ min Feed rate(F): 60 mm/min Milling depth: 10 mm Tools: T1 cutter Ø20 Tool path correction OFF Tool path correction ON and left Tool path correction ON and right | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circulare are anticlockweise |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numericlly Controlled<<< |  |
| Tasks: |  |
| - Program machining of work piece with G90 and for centre point path of the tool. - Use commands G00, G01, G02 and G03. - Enter the main program %8 and both subroutines L110 / L111 into your computer and simulate machining. - Consult your teacher for removing of any failures.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation speed(S): 2500 1/ min Feed rate(F): 60 mm/min Milling depth: 3 mm Tool: T1 cutter Ø6 | M03 spindle ON, clockwise rotation M30 main program STOP M17 subroutine END G01 straight line with given feed G02 circular are clockwise G03 circular are anticlockwise |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Please program machining of work piece with G90, with tool path correction and the circle – like depression as well as rectangle depression with corresponding cycles! - Use commands G00, G01, G02, G03, G40, G41, G42 and variables for the depressions! - Enter program %10 into your computer and simulate machining! - Consult your teacher for removing of any failures!. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation Speed(S): 1500 1/min 2500 1/ min Feed rate(F): 60 mm/min Milling depth: 6 mm Tools: T1 cutter Ø30, T2 cutter Ø12 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapit feed) G01 straight line with given feed G90 absolute position G02 circular are clockwise G03 circular are anticlockwise R01, R02, …. Parameters of cycles |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Tasks: |  |
| - Please program machining of work piece with G90 and for centre point path of the tool. - Use commands G0 and G1 as well as cycle polar multiple copying. - Enter program %11 into your computer and simulate machining. - Consult your teacher for removing of any errors.. |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 Rotation Speed(S): 1800 1/ min Feed rate(F): 60 mm/min Milling depth: 6 mm Tool: T1 Cutter Ø10 | M03 spindle ON, clockwise rotation M30 main program STOP G00 fast movement (rapid feed) G01 straight line with given feed G90 absolute position A 60° |

| #### Zespo Szkó Mechanicznych w Biaystoku |  |
| --- | --- |
| # >>>Computer Numerically Controlled<<< |  |
| Taskse: |  |
| - Please program machining of work piece with G90, with tool path correction and the circle – like depression as well as rectangle depression with corresponding cycles. - Use commands G00, G01, G02, G03, G40, G41, G42 and variables for the depressions. - Enter program %12 into your computer and simulate machining. |  |
| ## Input: |  |
| Tool – change - point (WWP): X=150; Y=100; Z=50 | ##### Select commands and parameters free (as you like) |

| # Answer sheet EN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | X= |  |  |  | Y= |  |  |  | Z= |  |  |  |  |  | Row part dimension |  |  |  |  |  |  |  |  |  |
| ## Tool store |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T 1 |  |  |  |  |  |  |  |  |  |  |  |  | T 4 |  |  |  |  |  |  |  |  |  |  |  |
| T 2 |  |  |  |  |  |  |  |  |  |  |  |  | T 5 |  |  |  |  |  |  |  |  |  |  |  |
| T 3 |  |  |  |  |  |  |  |  |  |  |  |  | T 6 |  |  |  |  |  |  |  |  |  |  |  |
| ## Program |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## N | G | X | Y | Z | I | J | T | S | F | M |  | ## N |  | G |  | X | Y | Z | I | J | T | S | F | M |