# Bilingualer Fachunterricht an berufsbildenden Schulen

# Unterrichtsmodul

Berufsfeld:  	Elektrotechnik

Beruf: 		Elektroniker

Capacitor

ACTION PROGRAMME OF THE EUROPEAN UNION

LEONARDO DA VINCI

PROJEKT NO.: 2002 LA 112 628 BILVOC

Language Competence Through

Bilingual Teaching at Vocational Colleges

###### Teaching Modules

Date of production			09/2005

Subject:					Electrical technology

Topic:					Capacitor

Title: 						Charge and discharge of the capacitor

Target Group: 				Any group of electrical engineering students

Prior Knowledge:				Basic of electrical engineering

Level of Language Skills:		Waystage (A2)

Follow - Ups:		 		Excercise with other electronic components

Objectives:					Capacitor in the technic

Benefits:	Rrelationhip between the voltage, curent and capacity

Teaching Module: capacitor

| Teaching Time (experience) | Contents | Activities | Media | Skills |
| --- | --- | --- | --- | --- |
| 45 minutes 15 minutes 60 minutes 30 minutes 20 minutes | Introduction and repetition to the topic Capacitor Formulation for the excercise Construction of the wiring and execution of this one Test according to formulation Preparing the diagrams evaluation | Pupil conversation Disskusion Work with the dictionary Teacher lecture Groups work with the formulation Construction of the wiring Noting down the results Groups draw the diagrams with your measurement results Evaluation and conclusion from the results | Board, worksheet dictionary transparency Excercise board paper transparency | Lessons conversation under use of the entry knowledge of the pupils Individual work Circuit arrangement with this Excercise board Individual work of the students Discussion of the results in the stdents conversation |
| 170 minutes |  |  |  |  |

Excercise Capacitors

# 1. Theory

Capacitors

Capacitors are the constructive elements, which are able to store electrical charges for example electrical energy.

In it’s simplest form a capacitor consists of two metal plates lying opposite each other.

Between these metal plates lies a ‘dilektric materia’, which prevents any electrical connection between the two plates. (Isolator)

When the capacitor is connected to a voltage an electrical field develops between the two plates.

‘Charging-time’ of a Capacitor.

To calculate the charging-time the value of the resistance on the capacitor and of the capacitor itself will be needed.

The level of voltage has no influence on the time it takes to charge.

Charging occurs quicker;

- The smaller the capacitor is.

- The smaller the circuit resistance is.

### Calculation of the charging-time

R * C is only a fraction of the charging time.
Time constant 
                       =R*C

After 5 Time constants the capacitor is charged

tE = 5 * 

Capacitors in a direct current circuit.

- When switched on the capacitor operates as a short circuit.

- The capacitor can store electrical energy.

- Obstructs the direct current

- charges and discharges itself in an E-function.

When switched on the electricity heavily increases.

After which the capacitor becomes high ressistance so that the electricity decreases and the voltage increases.

t

### Voltage:			Uc=U∗(1−e−tRL∗C)

### Current:  			Ic=URL∗e−tRL∗C

### When switched on  t= 0s  	Ico=URL

### Discharge capacitor

When discharge the voltage diminishes and the current rises. The current runs in the other direction.

### Voltage:			Uc=U∗e−tRE∗C

### current:

### When switched on    t= 0s  	Ico=URE

Exercises

1. Measure the voltage charge and discharge in 10 s measurements until the capacitor is charged.

Caution: Amps reverse the charge.

2. Measure the voltage charge and discharge. in 10s until the capacitor is charged.

3. Calculate the starting current.

4.  Show the diagrams for the voltage charge and discharge, current charge and discharge on a piece of millimeter squared paper in oblong format.

MEASURING DIAGRAM

C= 470 µF

RL= 47 kΏ

RE= 47 kΏ

Uq= 15 V

CHARGE

DISCHARGE

Result - Diagram

|  | L A D E V O R G A N G |  |  | E N T L A D E V O R G A N G |  |
| --- | --- | --- | --- | --- | --- |
| Zeit in Sek. | Ic in Ampere | Uc in Volt |  | Ic in Ampere | Uc in Volt |
| 0 | 0,000319149 | 0 |  | 0,000319149 | 15 |
| 10 | 0,000202951 | 5,46130509 |  | 0,000202951 | 9,53869491 |
| 20 | 0,000129059 | 8,93421996 |  | 0,000129059 | 6,06578004 |
| 30 | 8,20704E-05 | 11,1426916 |  | 8,20704E-05 | 3,85730835 |
| 40 | 5,21896E-05 | 12,5470875 |  | 5,21896E-05 | 2,4529125 |
| 50 | 3,31881E-05 | 13,4401611 |  | 3,31881E-05 | 1,55983893 |
| 60 | 2,11047E-05 | 14,0080782 |  | 2,11047E-05 | 0,99192185 |
| 80 | 8,53444E-06 | 14,5988814 |  | 8,53444E-06 | 0,40111865 |
| 100 | 3,4512E-06 | 14,8377935 |  | 3,4512E-06 | 0,1622065 |
| 120 | 1,39562E-06 | 14,9344061 |  | 1,39562E-06 | 0,06559393 |
| 140 | 5,64366E-07 | 14,9734748 |  | 5,64366E-07 | 0,02652522 |
| 160 | 2,28222E-07 | 14,9892736 |  | 2,28222E-07 | 0,01072641 |
| 180 | 9,22894E-08 | 14,9956624 |  | 9,22894E-08 | 0,0043376 |
| 200 | 3,73205E-08 | 14,9982459 |  | 3,73205E-08 | 0,00175406 |
| 240 | 6,10293E-09 | 14,9997132 |  | 6,10293E-09 | 0,00028684 |
| 300 | 4,03575E-10 | 14,999981 |  | 4,03575E-10 | 1,8968E-05 |

DIAGRAM

Evaluation

1. Use the time constant from the measured functions and calculate it from C.

2. Calculate the from Url=f(t) and add it to the diagram.

3. How do the charge and discharge times change when C; Re and Rl change.

4. Name possible uses of capacitors.

Words

Capacitor		Kondensator

capacity		Kapazität

dielectric material	Dielektrikum

charge			Ladung

discharge		Entladung